Two dimensional steady state heat transfer and temperature distribution over the annular plate with convection

Course Title – Finite Element Method

Course Code – ME 436/736

Submitted To

Sandeep singh

Department of Mechanical Engineering  IIT Indore

Submitted By

Chandan Maurya

(2502103017)

1. Introduction and Problem Statement

This project investigates the steady-state temperature distribution over a two-dimensional annular plate (a circular ring) using the Finite Element Method (FEM). The analysis incorporates Fourier's Law of Conduction and a Robin boundary condition for heat transfer via convection.

2. Geometry and Material Properties

| Inner radius | 0.5 m |
| --- | --- |
| Outer radius | 1 m |
| Thermal conductivity | 10 W/(m. K) |
| Heat transfer coefficient | 25 W/(m²·K) |
| Ambient temperature | 100 K |
| Internal radius temperature | 400 K |

3. Governing equation

## For 2D steady-state heat conduction with internal heat generation , the governing partial differential equation is the Poisson Equation

Weak Form

4.Methodology

clear;

clc;

close all;

% Data

ri     = 0.5;

ro     = 1.0;

k      = 10.0;

h      = 25.0;

Tinf   = 100.0;

Tinner = 400.0;

Q      = 0.0;

nr     = 5;

ntheta = 32;

% Mesh generation

rvec  = linspace(ri, ro, nr);

theta = linspace(0, 2*pi, ntheta+1);

theta(end) = [];  % remove duplicate

nodes = zeros(nr*ntheta,2);

nid   = zeros(nr, ntheta);

id = 0;

for i = 1:nr

for j = 1:ntheta

id = id + 1;

nodes(id,1) = rvec(i)*cos(theta(j));

nodes(id,2) = rvec(i)*sin(theta(j));

nid(i,j)    = id;

end

end

nNodes = size(nodes,1);

% Connectivity

elems = [];

for i = 1:nr-1

for j = 1:ntheta

jp = j + 1;

if jp > ntheta,

jp = 1;

end

elems = [elems; nid(i,j) nid(i,jp) nid(i+1,jp) nid(i+1,j)];

end

end

nElem = size(elems,1);

function [N, dN_dxi] = shapeQ4(xi, eta)

% Shape functions (1x4 vector)

N1 = 0.25*(1-xi)*(1-eta);

N2 = 0.25*(1+xi)*(1-eta);

N3 = 0.25*(1+xi)*(1+eta);

N4 = 0.25*(1-xi)*(1+eta);

N = 0.25 * [(1-xi)*(1-eta), (1+xi)*(1-eta),(1+xi)*(1+eta), (1-xi)*(1+eta)];

% Derivatives w.r.t. xi and eta

dN_dxi = 0.25 * [-(1-eta),  (1-eta), (1+eta), -(1+eta); -(1-xi),  -(1+xi),  (1+xi),   (1-xi)];

end

%% Global matrices

K = sparse(nNodes,nNodes);

F = zeros(nNodes,1);

% 2x2 Gauss

g = 1/sqrt(3);

gp = [-g -g; g -g; g g; -g g];

w  = [1 1 1 1];

% Element loop

for e = 1:nElem

conn = elems(e,:);

Xe = nodes(conn,:);

Ke = zeros(4,4);

fe = zeros(4,1);

for kq = 1:4

xi = gp(kq,1);

eta = gp(kq,2);

[N, dN_dxi] = shapeQ4(xi, eta);

J = dN_dxi * Xe;

detJ = det(J);

invJ = inv(J);

dN_dx = invJ * dN_dxi;      % 2x4

Ke = Ke + k * (dN_dx' * dN_dx) * detJ * w(kq);

if Q ~= 0

fe = fe + N' * Q * detJ * w(kq);

end

end

K(conn,conn) = K(conn,conn) + Ke;

F(conn)      = F(conn) + fe;

end

% Convection on outer boundary

gp1d = [-1/sqrt(3), 1/sqrt(3)];

w1d  = [1, 1];

for j = 1:ntheta

jp = j + 1; if jp > ntheta, jp = 1;

end

n1 = nid(nr,j);

n2 = nid(nr,jp);

x1 = nodes(n1,1);

y1 = nodes(n1,2);

x2 = nodes(n2,1);

y2 = nodes(n2,2);

L  = sqrt((x2-x1)^2 + (y2-y1)^2);

Je = L/2;

Ke_edge = zeros(2,2);

fe_edge = zeros(2,1);

for gq = 1:2

s = gp1d(gq);

N1 = (1-s)/2;

N2 = (1+s)/2;

Nedge = [N1 N2];

Ke_edge = Ke_edge + (Nedge'*h*Nedge) * Je * w1d(gq);

fe_edge = fe_edge + (Nedge'*h*Tinf)  * Je * w1d(gq);

end

idx = [n1 n2];

K(idx,idx) = K(idx,idx) + Ke_edge;

F(idx)     = F(idx)     + fe_edge;

end

%Dirichlet inner boundary

fixed = nid(1,:);

T = zeros(nNodes,1);

T(fixed) = Tinner;

F = F - K*T;

K(fixed,:) = 0; K(:,fixed)=0;

for ii = fixed

K(ii,ii) = 1;

F(ii) = T(ii);

end

% Solve

T = K\F;

% Results

fprintf('Min T = %.2f K,  Max T = %.2f K\n', min(T), max(T));

% Convert quads to triangles for plotting

tri = zeros(2*nElem,3);

for e = 1:nElem

c = elems(e,:);

tri(2*e-1,:) = c([1 2 3]);

tri(2*e,:)   = c([1 3 4]);

end

figure('Color','w');

trisurf(tri, nodes(:,1), nodes(:,2), T, 'EdgeColor','none');

view(2); axis equal; colorbar;

title('Temperature Distribution (K)');

xlabel('x (m)'); ylabel('y (m)');

5.Result and Discussion

The simulation result shows the temperature T across the x-y plane The plot confirms that the highest temperatures are maintained at the inner radius, and heat is smoothly transported to the outer convection boundary.

Maximum temperature= 400k ,Minimum Temperature=-316.82